IM50L35

5MP STARVIS HD Network Camera Module

- 1/2.8”5.0MP Progressive Scan CMOS

- 5.0MP HD resolution

- Embedded Web Server, support Internet Explore monitoring, configuration, upgrade

- Support sub domain, easily to implement DHCP (ADSL dial-up)

- Support the P2P on Android, IOS

- Support ONVIF2.6

- Support Alarm pre-record

- Standard SDK(Linux,Windows) ，easily to be integrated with other digital systems

Key Features：

Technical specifications:

| Image sensor |  |
| --- | --- |
| Sensor | 1/2.8” 5MP CMOS Sony IMX335 |
| Maximum resolution | 2592x1944@30fps |
| Minimum illumination | Color：0.05Lux B/W：0.01Lux |
| Camera |  |
| AGC | Auto/Manual |
| S/N ratio | ≥50dB（AGC OFF） |
| Shutter speed | 1/2 - 1/20 ,000s，Slow shutter support |
| Wide dynamic range | Digital WDR≥73.7dB |
| Exposure mode | Auto/ Manual/shutter mode |
| White balance | Auto/ Manual /ATW/ Indoor/Outdoor/ lamp |
| Day & Night | External control（IR Cut Filter） |
| Image resolution |  |
| Main stream | 2592*1944/1080P/720P@30fps 2560*1440/2480*1520/2304*1296/1080P/720P@25fps |
| Sub stream | D1/VGA/640*360/480*360/CIF@25fps； |
| General protocol |  |
| Network protocol | TCP/IP/HTTP/NTP/DHCP/SMTP/RTSP |
| ONVIF protocol | Standard ONVIF |
| Network interface |  |
| Wired | 1ch 10/100BaseT Ethernet，RJ45 interface |
| Other |  |
| Power supply | 12VDC |
| Size | 38x38mm |
| Operation temperature | -30°C ~85°C |
| Working humidity | ≤90%RH(non-condensing) |

| Interface | Pin | Pin name | Functional parameter |
| --- | --- | --- | --- |
| J1 | 1 | 12VDC | DC power input, voltage range 5V-13.6V |
|  | 2 | Gnd | Signal ground |
|  | 3 | ETH_LED | Network indicator light interface |
|  | 4 | IR | Soft photosensitive infrared |
|  | 5 | Eth_TX- | Adaptive network interface, physically receiving/transmitting signals (differential -) |
|  | 6 | Eth_TX+ | Adaptive network interface, physically receiving/transmitting signals (difference +) |
|  | 7 | Eth_RX- | Adaptive network interface, physically receiving/transmitting signals (differential-) |
|  | 8 | Eth_RX+ | Adaptive network interface, physically receiving/transmitting signals (difference +) |
|  | 9 | WL | White light program control |
|  | 10 | CDS_IN | Hard light input |
| J2 | 1 | IR-CUT＋ | IR-CUT signal + |
|  | 2 | IR-CUT－ | IR-CUT signal- |
| J3 | 1 | Alarm_in1 | Alarm input 1 |
|  | 2 | Alarm_in2 | Alarm input 2 |
|  | 3 | GND | Signal ground |
|  | 4 | Alarm_out1 | Alarm relay output 1 |
|  | 5 | Alarm_out2 | Alarm relay output 2 |
| J4 | 1 | Audio_In | Audio linear input interface, |
|  | 2 | Gnd | Signal ground |
|  | 3 | Audio_Out | Audio linear output interface |
|  | 4 | SYS_RST | Manual Reset |
|  | 5 | NC | / |
| J5 | 1 | 3.3V | 3.3V power supply |
|  | 2 | USB_DM | data D- |
|  | 3 | USB_DP | data D+ |
|  | 4 | GND | Signal ground |